STRATEGIC ENGINEERING & RESEARCH PROPOSAL

Evaluating Finalized NIST Post-Quantum Cryptography Ecosystem (ML-KEM, ML-DSA, FN-DSA) on Microcontroller Architectures.

Prepared By: 	Gonapa Shiva Teja (Electrical and 	Target : 	Friday Research

Computer Engineering) 	Review Sync

Shiv Nadar Institution of Eminence

Institution: 	Advisor: 	Prof. Amurtesh B.

(Deemed to be University)

G. Shiva Teja 	1

# 1. Executive Summary & Core Research Thesis

The deployment of Post-Quantum Cryptography (PQC) within the Internet of Things (IoT) landscape marks a critical paradigm shift. While cloud environments seamlessly absorb the computational overhead of newly standardized algorithms, resource-constrained edge devices face strict physical ceilings. This proposal outlines an implementation and benchmarking framework that targets a low-cost, low-power hardware design. By selecting the Raspberry Pi Pico / Pico 2 as the evaluation baseline alongside finalized NIST standards— ML-KEM for key exchange, paired with a comparative matrix of ML-DSA and FN-DSA for digital signatures— this research aims to uncover and solve the fine-grained trade-offs between local processing cycles and over-the-air communication energy.

## 2. Technical Architecture & Constraints Assessment

Moving away from standard single-board computers (which hide optimization flaws behind desktop-grade processors and high memory capacities), this project deliberately restricts the physical layer to a bare microcontroller environment.

| Component | Technical Baseline Profile | Research Context & Structural Relevance |
| --- | --- | --- |
| Hardware Board Raspberry Pi Pico / Pico 2 | - CPU: ARM Cortex-M0+ / Cortex-M33 - Clock: 133 MHz / 150 MHz - SRAM: 264 KB / 520 KB - Price: ₹400 - ₹700 | Forces evaluation under explicit memory boundaries. Eliminates the distortion of full Linux operating systems, exposing raw clock cycles, static storage footprints, and heap allocations. |
| Key Agreement ML-KEM (FIPS 203) | - Parameter: ML-KEM-512 - Public Key Size: 800 Bytes - Ciphertext Size: 768 Bytes | Establishes the foundational quantum-resistant asymmetric shared secret. Chosen as the mandatory baseline for cryptographic handshake initialization. |
| Primary Signature ML-DSA (FIPS 204) | - Parameter: ML-DSA-44 - Public Key Size: 1,312 Bytes - Signature Size: 2,420 Bytes | Evaluated as the primary digital signature standard. Serves as the high-security benchmarking baseline against which alternative mathematical lattices are compared. |
| Secondary Signature FN-DSA (FIPS 206) | - Parameter: FN-DSA-512 - Public Key Size: 896 Bytes - Signature Size: 666 Bytes | Evaluated specifically for over-the-air optimization. Features significantly smaller key footprints, directly lowering wireless payload demands. |

## 3. Architectural Pitfalls (Negatives) & Engineering Solutions

Deploying high-order lattice math onto a micro-architecture reveals critical bottlenecks. Below is the breakdown of technical problems alongside actionable solutions that form the core novelty of our proposed paper.

### Pitfall 1: SRAM Allocation Caps & Heap Exhaustion

CRITICAL RISK FACTOR:

The standard Raspberry Pi Pico features 264 KB of internal SRAM. ML-DSA-44 public keys and signatures require continuous blocks of memory during generation loops. If stack variables, network buffers, and temporary variables are allocated concurrently without planning, memory collisions occur, causing immediate chip crashes or hard faults.

ENGINEERING SOLUTION FOR OUR PAPER:

We will implement explicit zero-copy static memory management. Instead of runtime heap instantiation (via standard dynamic malloc calls), memory pools will be pre-allocated statically inside the .bss segment during compiling. Furthermore, intermediate mathematical buffers used during polynomial multiplication will be aggressively reused across separate processing routines.

### Pitfall 2: High Execution Latency & Rejection Sampling Cycles

CRITICAL RISK FACTOR:

ML-DSA uses a mathematical procedure called rejection sampling. The CPU must repeatedly loop through signing calculations until a validation check passes. On a 133 MHz processor lacking hardware vector extensions, this introduces erratic runtime execution spikes, causing timing variances that can leave the system vulnerable to timing side-channel attacks.

ENGINEERING SOLUTION FOR OUR PAPER:

Our framework will integrate assembly-optimized NTT (Number Theoretic Transform) primitives derived from the open-source pqm4 repository. By utilizing specific 32-bit assembly instructions native to the ARM Cortex-M architecture, polynomial multiplication cycles can be reduced by up to 40%, flattening execution latency and stabilizing variance loops.

### Pitfall 3: Floating-Point Math Complexity vs. Hardware Limits

CRITICAL RISK FACTOR:

While FN-DSA solves the over-the-air packet bottleneck with small keys, its core signing algorithm depends on complex floating-point calculations using the Fast Fourier Transform (FFT) over a specific mathematical space

(Falcon tree structure). The original Raspberry Pi Pico (Cortex-M0+) lacks a hardware Floating Point Unit (FPU), forcing it to run slow software emulations that consume huge amounts of CPU time and battery power.

ENGINEERING SOLUTION FOR OUR PAPER:

We will utilize a dual-pronged approach. For the older Pico, we will use a specialized fixed-point integer emulation library to replace floating-point operations. If we shift to the Pico 2, we will leverage its native ARM Cortex-M33 IEEE-754 FPU, compiling with strict hardware vector targets to accelerate the math loops.

## 4. Deep-Dive Literature Survey (With Direct Source References)

To defend this framework successfully during our group sync and to back up our academic claims, we utilize primary literature references that outline the state-of-the-art across industry and academia.

### Reference 1: The NIST Standardization Baseline

Context: National Institute of Standards and Technology (NIST) Federal Information Processing Standards (FIPS) Publications 203, 204, and 206. This is the definitive global authority confirming that Kyber, Dilithium, and Falcon are the finalized standards, rendering older candidate algorithms obsolete.

URL:

https://csrc.nist.gov/projects/post-quantum-cryptography/selected-algorithms-fips-203-204-206

### Reference 2: The Core Framework For Microcontrollers (pqm4)

Context: Kannwischer et al., "pqm4: Benchmarking Post-Quantum Cryptography on ARM Cortex-M4". This foundational framework establishes how to properly measure execution cycles and memory footprints on constrained ARM profiles. It forms the exact technical precedent for our benchmarking methodology on the Raspberry Pi Pico.

URL: https://github.com/mupq/pqm4

### Reference 3: Key Size vs. Network Fragmentation Analysis

Context: Schöffel et al. (2022), "Post-Quantum Cryptography in IoT: A Performance Evaluation." This study highlights how large PQC signature fields exceed maximum transmission unit (MTU) limits in low-power networks, leading to heavy packet fragmentation and severe wireless radio battery drain. This paper provides the direct motivation for our comparative evaluation of FN-DSA.

URL: https://doi.org/10.1109/IOTM.001.2100115

### Reference 4: Floating-Point Optimizations for Falcon

Context: Recent 2024-2025 implementations focusing on running Falcon signatures on Cortex-M architectures without FPUs via fixed-point math adaptations. This provides the primary technical blueprint for resolving our third architectural pitfall.

URL: https://openquantumsafe.org/

Thank You