To give you an absolute advantage in your Friday group meeting, here is a highly structured, side-by-side comparison matrix. It maps out exactly why we are choosing our specific hardware and algorithms over the alternative options, outlining the technical and economic trade-offs.

🛠️ Part 1: Hardware Selection (Why Pico over Raspberry Pi?)

| Feature | Selected: Raspberry Pi Pico / Pico 2 | Alternative: Standard Raspberry Pi (Pi 4 / 5) | Why our choice wins for research |
| --- | --- | --- | --- |
| Device Type | Microcontroller (Runs bare-metal C/C++ code directly on the chip). | Single-Board Computer (Runs a massive Linux Operating System). | Microcontrollers represent real-world IoT edge nodes. Running security on Linux doesn't show edge bottlenecks. |
| RAM Ceiling | 264 KB to 520 KB SRAM | 2 GB to 8 GB LPDDR4 | Strict RAM limits force us to optimize static layouts and analyze stack allocations, creating deep research value. |
| CPU Profile | 133 MHz – 150 MHz ARM Cortex-M | 1.5 GHz – 2.4 GHz ARM Cortex-A | Exposes raw cryptographic clock cycles instead of hiding inefficiencies behind desktop-grade processors. |
| Cost / Budget | ~₹400 – ₹700 ($5 – $8) | ~₹3,500 – ₹6,000+ ($45 – $80) | Incredibly budget-friendly. It runs via a single USB cable connected to your laptop with no extra accessories needed. |

🔐 Part 2: Key Exchange Selection (Why ML-KEM over FrodoKEM?)

| Feature | Selected: ML-KEM (Kyber) | Alternative: FrodoKEM | Why our choice wins for research |
| --- | --- | --- | --- |
| NIST Status | Officially Finalized Standard (FIPS 203) | Kept only as a backup alternative candidate. | Reviewers want evaluations of current industry standards, not backup options. |
| Math Structure | Structured Lattices (Module-LWE) | Unstructured Lattices (Plain LWE) | Structured math runs significantly faster on tiny embedded chips. |
| Public Key Size | 800 Bytes (ML-KEM-512) | ~9,600 Bytes (FrodoKEM-640) | ML-KEM easily fits inside the Pico's 264 KB RAM. FrodoKEM keys are 12x larger, causing severe packet fragmentation over wireless links. |
| Processing Speed | Extremely fast on ARM architecture. | Heavy compute loops that drain battery power. | Keeps processing time minimal, protecting battery life. |

✒️ Part 3: Digital Signature Selection (Why ML-DSA vs. FN-DSA?)

Instead of choosing one signature algorithm over another, we are selecting both to compare them. Here are the contrasting differences that make their comparison a perfect foundation for a paper:

| Feature | Candidate A: ML-DSA (Dilithium) | Candidate B: FN-DSA (Falcon) | The Research Comparison Value |
| --- | --- | --- | --- |
| Key & Signature Size | Larger (Public Key: 1,312B / Signature: 2,420B) | Ultra-Small (Public Key: 896B / Signature: 666B) | The Network Trade-off: FN-DSA prevents wireless network packet fragmentation, while ML-DSA strains network packet capacities. |
| CPU Math Load | Moderate (Uses integer-based Rejection Sampling). | Heavy (Requires complex Floating-Point math). | The Processor Trade-off: ML-DSA is easier on simpler CPUs. FN-DSA requires an advanced hardware Floating Point Unit (FPU) to run efficiently. |
| Runtime Latency | High variance (Loops until verification passes). | Constant, predictable timing footprints. | Allows us to benchmark local CPU loop variations against over-the-air transmission costs. |

💬 The Ultimate Argument for Your Group

If your team asks why you chose this exact setup, read or paste this core argument:

"If we use a standard Raspberry Pi, we are just testing code on a mini-desktop, which lacks research novelty. By using the Raspberry Pi Pico, we face real-world memory limits (under 512 KB RAM), turning our experiment into a true resource-constrained study.

For cryptography, Frodo KEM

is an outdated path because its 9.6 KB keys will easily overload our microcontroller's RAM and fragment our network packets. Instead, pairing the finalized ML-KEM standard with a direct comparison between ML-DSA and FN-DSA gives us the perfect narrative for a publication: we are analyzing whether it is better for an IoT device to save battery by reducing network packet size (FN-DSA) or by reducing CPU computing stress (ML-DSA)."